GIANT UPS

Specification

- SELF TEST:

Self test each time switched on.

When UPS is turned on, it makes self-test to provide a reliable operation and prevent any damage caused.

BATTERY PROTECTION:

Auto self-test and discharge protection.

- DC START CAPABILITY:

The UPS can be activated without utility power.

- UPS POWER SUPPLIED OUTLETS:

Line-Interactive UPS+AVR

Allows 75% to 125% input voltage range-best protection for computer equipment.

With AVR function (boost & buck), UPS provides a stable power to the loads as the input voltage fluctuates.

- ELECTRICAL PROTECTION:

Automatic cut off on short circuit and Over Load.

- AUDIBLE ALARM:

Battery Back-up: slow beeping (10 seconds) Battery Low and Overload or AC Overload: Rapid beeping (1 Second)

- CPU MICRO PROCESS CONTROL, USE WITH STABILITY.

- GREEN FUNCTION:

If the battery is discharging under no load, will shut down automatically to keep the span life of battery

| UPS MODEL |  | GT-500 |
| --- | --- | --- |
| Capacity |  | 500VA |
| Input | Voltage | 110V(80-140V) / 220V(160-280V) |
|  | Frequency | 50Hz±5% / 60Hz±5% |
| Output | Voltage (Battery Back-up) | 110V±10% / 220V±10% |
|  | Frequency (Battery Back-up) | 50Hz±0.5% / 60Hz±0.5% |
|  | Regulation Wave | Stepped approximation to Sine wave |
| Battery | Type | Sealed maintenance free lead-acid battery |
|  | Recharge Time | < 4 hours to 90% of full capacity |
|  | Back-up Time | 5 - 10 Min |
|  | Numbers | 12V 4.5AH x 1 |
| Approval | Safety | CE / CNS |
| Environment | Operating Temperature | 0 ~ 35C (32 ~ 95F) |
|  | Relative Humidity | 0 ~ 95% Non-Condensing |
| Surge Energy Rating |  | 240 Joules |
| Normal Mode Noise |  | Full Time EMI/RFI Filtering |
| Phone Jack |  | For Modem/Fax/Network |
| Protection |  | Short & Overload; Battery Low Cut off |
| Outlets |  | IEC or USA |
| Physical | Weight (Kg) | 4 kgs |
|  | Dimension W x D x H (mm) | 107 x 230 x 119 |
| Installation method |  | Wall mounted, or flat-put |